QCar2 Autonomous Vehicle Project

We are building a self-driving miniature car from scratch.A fully autonomous vehicle that can see the road, read traffic signs, avoid obstacles, plan its own route, and drive itself from point A to point B without any human touching the controls. We have defined our own set of test cases. We want to prove that our system works in all of these situations:

For lane detection the car must follow a yellow tape lane on a black surface in straight sections, in curves, in S-curves, when part of the lane is covered, when lighting is harsh, when only one lane boundary is visible, and when the lane temporarily disappears and the car must remember where it was. Every one of these is a real scenario that happens on real roads and breaks simple systems immediately.

For traffic understanding  the car must correctly read and respond to stop signs, speed limit signs, turn direction signs, no-entry signs, and pedestrian crossing signs at different distances and angles. It must not just detect that a sign exists but actually change its behavior based on what the sign says.

For obstacle handling  the car must stop for obstacles in its path, wait, check if they have moved, and if the path is clear resume driving. If the obstacle is permanent it must calculate an alternative path around it and continue to the destination.

For full integration  the car must complete a full course that has lanes, traffic signs, obstacles, and intersections  all active at the same time  entirely without human help from start to finish.

What We Have Already Achieved:

In the first phase of this project we have accomplished several significant technical milestones that form the foundation of everything that comes next.

We got all four cameras working simultaneously. The cameras use a special interface called CSI that requires a GPU display context to initialize. Since our car runs headlessly over a remote connection without a physical monitor, this was failing silently every time. We identified the root cause deep in NVIDIA's camera driver stack, developed a two-line fix involving environment variable management, and validated it with all four cameras streaming live simultaneously.

We have a complete lane detection pipeline running in real time on the car's camera feed. The car can see yellow tape on a black surface, calculate exactly how far it is from the center of the lane in pixels, and generate steering commands to correct its position. We have a PID steering controller tuned and working. PID stands for Proportional-Integral-Derivative  it is the mathematical formula that translates lane position error into steering commands.

We have LiDAR obstacle detection working. The 360-degree laser scanner reads distances 360 times per second. Our software filters the front region of those readings and triggers an emergency stop when anything comes within 1.5 meters of the front of the car. The car stops, waits for the obstacle to clear, counts 5 frames after it clears, then resumes driving mimicking exactly how real autonomous vehicles handle unexpected obstacles.

We have YOLOv8 — one of the world's best real-time object detection AI models — running on the car's GPU. It can detect traffic signs, people, vehicles, and obstacles in real time from the RealSense camera at 30 frames per second.

What We Are Doing Right Now:

We are in the process of calibrating the lane detection system for our specific test track and tuning the system parameters so the car can complete a full autonomous lap. The structure of the codebase is being professionalized into a proper software package with clean separation between hardware management, computer vision, and control  exactly how production autonomous vehicle software is organized at companies like Waymo and Cruise.

SDV QCar2 Development Log

The vehicle uses a camera, an Intel RealSense depth sensor, a LiDAR, an IMU, and wheel encoders. The software stack runs on ROS2 and uses YOLOv8 for real-time object detection and EasyOCR as a fallback sign reader. A state machine controls vehicle behavior forward movement, left turn, right turn, and stop based on what the vehicle detects.

Sensor data comes in from the camera, depth sensor, and LiDAR every cycle. The RGB frame is passed to a YOLOv8 model which classifies and localizes objects. If YOLO does not detect a relevant sign, an EasyOCR fallback reads text from a cropped region of the frame. Detections from both systems feed into a counter-based debounce system. Once a detection is confirmed across multiple frames, the vehicle state machine switches  for example, from moving forward to turning left. The vehicle executes the action and returns to the default forward state.

### Work Done — My Contributions

Task 1: OCR Detection Fix The previous OCR implementation was running on the full camera frame and producing noisy, unreliable detections. I analyzed the issue and implemented a center ROI crop so OCR only processes the relevant part of the image. I also added confidence-based filtering to discard weak detections and integrated OCR output into the same debounce counter used by YOLO, so both systems work together instead of independently.

Task 2: State Machine Understanding and Validation I traced through the complete state machine logic — MOVE_FORWARD, TURN_LEFT, TURN_RIGHT, STOPPED — and validated that state transitions were happening correctly based on detection triggers. I confirmed the permanent stop behavior works as intended: once a stop sign is confirmed, the car does not move again.

Task 3: Parameter Tuning I set and tested key parameters including detection confidence thresholds, sign distance ranges, trigger frame counts, and turn duration. Each parameter directly affects how early and how accurately the car responds to signs.

Task 4: Latency Identification I identified that the pipeline has high end-to-end latency, primarily from depth camera processing and OCR. I documented this as a known issue and identified TensorRT optimization and OCR frequency reduction as the next steps.

### Approach-

For OCR noise: I applied a center ROI crop before passing the frame to EasyOCR. The center of the frame is where the sign is most likely to appear, so cropping to 60% of the frame eliminates most background noise. I also added a confidence threshold of 0.55 so only high-confidence text is accepted. I chose this approach because it is computationally cheap and requires no model changes.

For latency: I reduced OCR frequency to once every 5 frames. This is the most direct way to cut OCR-related latency without disabling it entirely. The longer-term fix is TensorRT optimization for YOLO, which can give 5 to 10 times faster inference on the Jetson GPU.

For YOLO and OCR coordination: I integrated OCR detections into the same det_counter dictionary that YOLO uses. This means both sources increment and decrement the same counters, and the state machine sees a unified, consistent detection signal regardless of which system detected the sign.

## Day 2 -

### Objective of the Day

The primary goal was to perform a full system audit of the QCar2, identify all critical environment issues causing poor performance, and resolve them so that the hardware is operating at full capability before any further software development continues.

### Findings

A complete inspection of the QCar2 system was carried out. The following was confirmed about the hardware and software environment.

Hardware confirmed:

- Board: NVIDIA Jetson AGX Orin Developer Kit

- JetPack: R35, Revision 6.0 (August 2024)

- RAM: 30GB available

- Storage: 185GB free

- CPU: 8-core ARMv8

- CUDA: 11.4 installed

Software confirmed:

- OS: Ubuntu 20.04 LTS

- Python: 3.8.10

- ROS2: Humble

- Quanser package: qcar2-autonomy installed

- Autoware: Present on the system

### Problem 1 — CUDA Not Accessible by PyTorch

What was wrong: PyTorch version 2.4.1 was installed as a CPU-only build. Even though CUDA 11.4 was correctly installed on the Jetson, PyTorch could not detect the GPU. This meant all AI inference  including YOLOv8  was running on CPU, which is the primary cause of the high latency identified at the end of Day 1.

How it was solved: The CPU-only PyTorch was removed. The NVIDIA-official Jetson-optimized PyTorch wheel was downloaded directly from NVIDIA's distribution servers and installed. This build is specifically compiled against CUDA 11.4 for the ARM architecture of the Jetson.

### Problem 2 — Torchvision Not Available for Jetson

What was wrong: The standard pip torchvision packages are compiled for x86 (Intel/AMD) architecture. The Jetson uses ARM (aarch64), so prebuilt wheels either failed to install or installed an incompatible version.

How it was solved: Torchvision was compiled from source directly on the Jetson using the v0.16.0 source branch, matched to the installed PyTorch version. All required build dependencies were installed before compilation. The build took approximately 15 minutes on the Jetson hardware.

### Problem 3 — CUDA Path Not in System Environment

What was wrong: Even though CUDA 11.4 was installed, the system PATH and LD_LIBRARY_PATH did not include the CUDA binary and library directories. Commands like nvcc were not resolving correctly from the terminal.

### Problem 4 — Broken .bashrc Entries Causing Source Errors

What was wrong: During initial setup attempts, incorrect ROS2 workspace paths were written to .bashrc. These pointed to directories that did not exist, causing errors every time a new terminal opened.

How it was solved: The incorrect lines were identified and removed using sed commands. The correct paths were found by locating the actual setup.bash files on the filesystem, and the .bashrc was rewritten with verified paths only.

Correct paths confirmed:

- ROS2 Humble: /opt/ros/humble/setup.bash

- Quanser workspace: /home/nvidia/Documents/Quanser/5_research/sdcs/qcar2/ros2/install/setup.bash

- Custom ROS2 workspace: /home/nvidia/ros2/install/setup.bash

### Problem 5 — PyTorch Being Overwritten by pip

What was wrong: When installing other packages such as OpenCV or ultralytics, pip's dependency resolver automatically pulled in a newer but CPU-only version of PyTorch from PyPI, overwriting the CUDA-enabled Jetson build.

How it was solved: A pip constraint file was created that pins PyTorch to the specific Jetson-compatible version. This prevents pip from ever replacing it during future installations.

### SDV Workspace Created

A structured project workspace was created on the Desktop for all future development:

### Jetson Performance Mode Enabled

The Jetson AGX Orin was set to maximum performance mode. This unlocks all CPU cores, maximizes GPU clock speed, and removes power throttling — directly reducing inference latency.

### Sensor Verification

After sourcing the correct Quanser workspace, the full ROS2 sensor stack was found to already be running. The following sensors were confirmed live:

| Sensor | ROS2 Topic | Status |
| --- | --- | --- |
| LiDAR | /scan | Live |
| IMU | /qcar2_imu | Live |
| Odometry (EKF filtered) | /odometry/filtered | Live |
| Map | /map | Live |
| Battery | /qcar2_battery | Live |
| Camera | Not yet visible in topics | Pending |

The IMU confirmed correct gravity reading of 9.87 m/s². The LiDAR was detecting objects in the environment. The Nav2 full navigation stack was already active through the Quanser launch files.

### Key Discovery -

During the launch file inspection, it was found that Quanser has already provided working implementations of several autonomy components as part of the qcar2-autonomy package:

- lane_detector — lane detection node

- yolo_detector — YOLO-based object detection node

- traffic_system_detector — traffic sign/light detection

- path_follower — path following controller

- trip_planner — route planning node

### Issues at End of Day 2

- Camera node not yet publishing to ROS2 topics. The csi and rgbd nodes need to be tested individually to identify the correct launch method.

- YOLO and lane detector nodes exit with failure when run standalone, likely because they depend on the camera topic being active first.

## Day 3 to 6 -

### Objective of the Day

Following the CSI camera issue identified at the end of Day 2, this session focused on diagnosing the root cause of the failure, attempting all available software-level fixes, and establishing a working camera pipeline using the Intel RealSense as the primary sensor while the CSI issue was being investigated.

### RealSense Camera Activation

The Intel RealSense D435 was brought online first to unblock perception development. The rgbd node was launched using the Quanser ROS2 stack and the following topics were confirmed active.

Confirmed topics:

- /camera/color_image — 1280x720, BGR8 encoding

- /camera/depth_image — depth data active

Confirmed via topic echo: height: 720    width: 1280  encoding: bgr8

The RealSense was confirmed as the primary camera for all perception work going forward.

### CSI Camera Investigation -

A systematic attempt was made to identify and fix the root cause of the CSI camera failure. Every approach was attempted in sequence.

Attempt 1 — Quanser ROS2 Node (All Resolutions and Framerates)

The csi node from the qcar2_nodes package was run with every native resolution and framerate combination supported by the IMX219 sensor: 820x410 at 120fps, 820x616 at 80fps, and 1640x1232 at 80fps. All attempts produced the same error:

Error opening device. result: -1074.The video format is not supported.

Camera IDs 0 through 3 were all tested individually as well, including after correcting the camera mapping from the official Quanser documentation (front=2, rear=1, left=3, right=0). None succeeded.

Attempt 2 — PAL Library Camera2D Direct Access

The Quanser Python library pal.utilities.vision.Camera2D was found at /home/nvidia/Documents/Quanser/0_libraries/python/ and added to PYTHONPATH. When tested directly, the camera appeared to open and returned a frame shape of (616, 820, 3) but all pixel values were 0 — blank frames. The frame rate was approximately 0.5fps instead of the expected 80fps.

Attempt 3 — QCarCameras Class

The pal.products.qcar.QCarCameras class was tested as the official Quanser camera interface. After resolving a file permission error on qcar_config.json, the class imported successfully. However, all four camera read flags returned False and pixel values remained 0. This class uses Camera2D internally, so it hit the same underlying issue.

Attempt 4 — IMX219 Kernel Module Loading

The IMX219 kernel module was found at /lib/modules/5.10.216-tegra/kernel/drivers/media/i2c/nv_imx219.ko but was not loaded. It was manually loaded using sudo modprobe nv_imx219. The nvargus-daemon was restarted afterward. Cameras still returned blank frames.

Attempt 5 — Device Tree Overlay

The device tree overlay file tegra234-p3737-camera-imx219-overlay.dtbo was found in /boot. It was not applied to the boot configuration. The /boot/extlinux/extlinux.conf was backed up and the overlay was added under the primary boot label. After reboot, dmesg confirmed all four IMX219 sensors bound successfully at kernel level:

imx219 30-0010: tegracam sensor driver:imx219_v2.0.6 — bound

imx219 31-0010: tegracam sensor driver:imx219_v2.0.6 — bound

imx219 32-0010: tegracam sensor driver:imx219_v2.0.6 — bound

imx219 33-0010: tegracam sensor driver:imx219_v2.0.6 — bound

imx219 34-0010: board setup failed (5th slot, does not exist on QCar2 — expected)

Despite sensors being bound, pixel values remained 0. The kernel sees the hardware but the camera service cannot stream frames.

Attempt 6 — EGL Display Context Fix

The Argus daemon requires an EGL display context to create frame consumers. Since the QCar2 is accessed via Remote Desktop with no physical monitor attached, EGL initialization fails. The following approaches were tried to provide a display context:

- Setting DISPLAY=:0 and restarting nvargus-daemon

- Installing xvfb and creating a virtual display at :1

- Starting nvargus-daemon manually with sudo -b DISPLAY=:0

All failed with: Failed to initialize EGLDisplay and Failed to get default display.

Attempt 7 — Direct GStreamer Pipeline

A GStreamer pipeline using nvarguscamerasrc was tested directly. The pipeline started but produced the error:

(Argus) Error BadParameter: Failed to create FrameConsumer

/dev/camera/video4: No such file or directory

NvPclHwGetModuleList: No module data found

This revealed the core issue: the /dev/camera/ device nodes do not exist on this system.

Attempt 8 — OpenCV V4L2 Direct Access

OpenCV was used to open /dev/video0 directly via cv2.CAP_V4L2, bypassing the Quanser SDK and Argus entirely. The device opened but cap.read() timed out. The V4L2 device exists but cannot stream without the camera middleware layer.

Attempt 9 — v4l2-ctl Raw Bayer Capture

v4l2-ctl was used to attempt a raw Bayer frame capture directly from /dev/video0 with the native RG10 format. The command ran indefinitely without producing any output. Confirmed the same middleware dependency.

Attempt 10 — QUARC camsock Unix Socket

The file /tmp/camsock was found to be an active Unix socket owned by the QUARC camera service. The libraries libnvscf.so and libsensors_hal.nvs.so were found to reference this socket. libnvscf.so exports the function scf::getCameraDriver() which is the C++ camera driver interface used by QUARC. However, this library internally uses eglGetDisplay and eglGetProcAddress, meaning it has the same EGL dependency as Argus and cannot be called on a headless setup.

Attempt 11 — Probe and Observer Local Integration

The Quanser Probe and Observer classes from pal.utilities.probe were used together running locally on the QCar2, with Probe(ip='localhost') pointing to a local Observer thread. Connection succeeded (Connected: True) but all camera flags remained False and pixel values were 0. The Probe/Observer system is a network visualization layer on top of Camera2D and does not change the underlying driver behavior.

### Root Cause — Confirmed

All 11 approaches lead to the same conclusion: the /dev/camera/ device nodes do not exist on this QCar2 unit. These nodes are created by Quanser's custom modified JetPack image. The unit has the stock NVIDIA JetPack installed, which does not include Quanser's camera middleware layer that sits between Argus and the application layer.

The IMX219 sensors are physically present and detected at boot. The kernel module and device tree overlay are correct. The missing component is exclusively the Quanser-specific camera device node layer.

### Camera Bridge — RealSense + CSI (Partial)

In parallel with the CSI investigation, a ROS2 Python node camera_bridge.py was written and placed inside the sdv_perception package at:

~/Desktop/SDV_workspace/src/sdv_perception/sdv_perception/camera_bridge.py

The node opens all four CSI cameras using Camera2D with a 2-second stagger between each to prevent segmentation faults from simultaneous initialization. It publishes to four ROS2 topics. The RealSense node was run separately via ros2 run qcar2_nodes rgbd.

Topics confirmed active: /sdv/camera/front   /sdv/camera/left  /sdv/camera/rear

/sdv/camera/right    /camera/color_image  (RealSense RGB)

/camera/depth_image  (RealSense Depth)

The CSI topics published blank frames due to the known driver issue. The RealSense topics published valid data.The camera_bridge frame rate for CSI was measured at approximately 0.3–0.5fps due to the Quanser SDK blocking read calls. This is a hardware driver issue, not a threading issue.

## Day 7-

### Objective of the Day

Continued investigation into the CSI camera issue with a focus on deeper library-level analysis. A new discovery was made: when a physical HDMI monitor is connected to the QCar2, the CSI camera windows open. This confirmed the root cause conclusively and shifted the investigation toward understanding the exact dependency chain.

### Key Discovery — HDMI Monitor Resolves EGL

When a physical HDMI monitor was connected to the QCar2, four CSI camera windows opened and displayed output. This confirmed that the failure is not a hardware sensor issue or a driver version issue. The Argus camera service requires a real GPU-backed EGL display context to create its frame consumer pipeline. A physical monitor connected to the HDMI port satisfies this requirement. Remote desktop and virtual displays do not because they are software-backed and do not initialize the NVIDIA GPU's display engine.

Conclusion: CSI cameras function correctly when a physical HDMI monitor is connected. For headless deployment, Quanser's custom JetPack image is required to create the /dev/camera/ device nodes that bypass Argus entirely.

### Deep Library Analysis — libquanser_media.so

A full analysis of the Quanser media library was performed to understand the exact camera access path.

Key findings from library inspection:

The library libquanser_media.so contains both an Argus path and a V4L2 path. It links against libnvargus_socketclient.so for the Argus path and libv4l2.so.0 for the V4L2 path. It also contains the YUV420 to BGR888 conversion functions built in, meaning it handles the full image pipeline internally.

The Argus path calls eglGetDisplay through libnvargus_socketclient.so. On a headless system this returns null, causing all downstream Argus operations to fail. The library attempts Argus first and does not automatically fall back to V4L2 for CSI sources.

The V4L2 path in the library references /dev/video%u device nodes directly, but these are used for a different class of devices (USB cameras, encoder devices) not for the CSI IMX219 sensors.

Additional discovery: The file /tmp/camsock is an active Unix socket used by the QUARC camera service, and /usr/lib/firmware/quanser/QCar2.qfl is the Quanser firmware file for this specific platform. These confirm that Quanser's custom camera stack is partially present but the device node layer is missing.

Attempt 1 — Direct VideoCapture API Testing

The Quanser VideoCapture class from quanser.multimedia was tested directly against all four camera IDs and all three supported resolutions. All 12 combinations returned error code -1074. Error -1074 in the Quanser SDK means the video server is not running or cannot open the device.

Attempt 2 — Quanser Video Server Search

A search was conducted to find and start the Quanser video server process. The QUARC target manager service was found running and listening on port 17000. Port 16999 was also found listening. The libquanser_media.so library and the quanser.multimedia Python module were both confirmed present. No standalone video server process was found that could be started to unblock CSI access.

Attempt 3 — Fake EGL Preload

A fake EGL shared library was compiled in C that returns a non-null display handle from eglGetDisplay and a successful result from eglInitialize. This was preloaded using LD_PRELOAD before launching the Python camera code. The fake EGL successfully intercepted the call but the Argus session creation failed at a deeper level because it performs additional EGL validation that the stub did not implement. The approach was abandoned as increasing complexity without a clear path to success.

### Final Decision on CSI Cameras

For all future development, the Intel RealSense D435 is used as the primary camera. It provides 1280x720 RGB at approximately 16fps and full depth data, which is superior to CSI for the perception use case because it adds the third dimension.

The practical workaround established: when CSI camera output is needed for testing or demonstration, a physical HDMI monitor is connected to the QCar2.

## Day 8 -

### Objective of the Day

Build and launch the full SDV Perception Pipeline v2.0 on the QCar2 with real-time GPU-accelerated object detection, depth fusion, lane detection, and LiDAR processing, all running as a ROS2 node publishing live annotated output.

### torchvision C++ Ops — Root Cause and Fix

Problem: The torchvision installation present on the system was built without CUDA support. The torchvision.ops.nms() function, which YOLOv8 requires for non-maximum suppression, failed with RuntimeError: Couldn't load custom C++ ops. This meant YOLO inference was breaking at the post-processing stage even though the Jetson GPU was active.

Root cause: Two conflicting torchvision installations were present simultaneously — one in /usr/local/lib/ and one in /home/nvidia/.local/lib/. The .local/ version was a pip-installed CPU-only build that lacked the compiled C++ extensions. Python was picking up the wrong one.

How it was fixed: Both conflicting installations were removed. torchvision was rebuilt from the v0.16.0 source branch with FORCE_CUDA=1 and CUDA_HOME=/usr/local/cuda-11.4 explicitly set. This compiled the full C++ extension including CUDA-accelerated NMS. The resulting wheel was installed to the user path. After this, NMS ran on GPU and returned correct results.

### ROS2 Package Build

The sdv_perception ROS2 package was rebuilt using colcon build to incorporate all source-level fixes. The workspace install was sourced and the package was confirmed discoverable by ROS2.

### Perception Pipeline Launch

The full perception pipeline was launched using the ROS2 launch file. The pipeline initialized successfully with the following confirmed output.

The RealSense node was launched in a separate terminal. Once /camera/color_image began publishing, the perception node started processing frames automatically.

### Pipeline Architecture -

The perception node subscribes to three input topics and publishes four output topics.

Inputs:

- /camera/color_image — RGB frames from RealSense for YOLOv8 detection

- /camera/depth_image — depth frames from RealSense for 3D position calculation

- /camera/color_image — same RGB feed used for lane detection (CSI front camera pending)

Outputs:

- /sdv/perception/detections — JSON list of detected objects with class, confidence, and 3D distance

- /sdv/perception/lane — lane offset in meters as a float array

- /sdv/perception/threat — threat level string for obstacle proximity

- /sdv/perception/annotated — annotated image with detection boxes, distances, and lane overlay

Pipeline modules active:

- YOLOv8 on GPU with confidence filtering

- Depth fusion using RealSense pinhole model to compute real-world distance per detection

- Lane detection using HLS color space, bird's eye perspective transform, and polynomial curve fitting with temporal smoothing

- LiDAR processor scanning a front 60-degree cone for obstacle proximity

### First Live Detection Result

The perception pipeline produced live annotated output with the following observed detections:  FPS: 2.1  Latency: 37.4ms  Objects detected: 3 person — 95% confidence 1.0m       person — 63% confidence — 0.7m

This confirmed that:

- YOLOv8 is running on the Jetson Orin GPU

- Depth fusion is computing real-world distances per detection in meters

- The full pipeline runs end-to-end at 37.4ms latency

- Annotated video output is publishing to /sdv/perception/annotated

The 2.1fps frame rate is below target and is attributed to the RealSense node publishing at approximately 16fps while the perception node processes at a lower rate due to the full pipeline overhead. TensorRT optimization is identified as the next step to bring this to 15–30fps.

### Significance of This Milestone

The perception layer is the foundational requirement for all subsequent autonomous driving modules. Object detection with real-world distance gives the vehicle the ability to make safety-critical decisions — emergency braking, safe following distance, obstacle avoidance. Without this working reliably, no planning or control logic can function correctly.

### Days 9–11 — Lane Detection and CSI Camera Fix

Initial lane detection used the RealSense RGB camera. The pipeline converted frames to HLS color space, applied color masking for the lane markers (black lines on white table surface during lab testing), performed ROI masking with a trapezoid, applied a bird's-eye perspective transform using calibrated source and destination points, ran histogram-based lane base detection, executed a 9-window sliding window search, and fit a second-degree polynomial using numpy polyfit with temporal exponential moving average smoothing across 8 frames. The lane offset was published to /sdv/perception/lane and consumed by the control node.

Critical CSI Camera Fix — Root Cause and Resolution:

All four CSI cameras were non-functional when running over SSH or RDP. The error appeared as "video format not supported" or blank frames with max pixel value of zero. After investigation, the root cause was identified at the EGL display layer.

When Quanser's libquanser_media.so loads, it calls into libnvargus_socketclient.so which internally calls eglGetDisplay() to allocate hardware video memory on the Jetson GPU. This call requires an authorized X11 display context. When running over SSH or RDP without a physically connected display, the DISPLAY and XAUTHORITY environment variables either point to a session that the GPU's EGL layer is not authorized for, or are missing entirely. As a result, nvargus-daemon rejects the camera format request and returns blank frames.

The fix was two-part. First, a dummy HDMI dongle was physically connected to the QCar2's HDMI port. This makes the GPU believe a monitor is connected and enables proper GPU memory context initialization. Second, before calling QCarCameras(), the DISPLAY and XAUTHORITY environment variables are explicitly stripped using os.environ.pop() so that nvarguscamerasrc is forced into headless NVMM memory capture mode, bypassing the display authorization check entirely. After camera initialization, DISPLAY is restored so that OpenCV's imshow can still render the output window.

This fix was applied to the camera_bridge.py in the ROS2 perception node and to all standalone scripts. After the fix, all four CSI cameras initialized successfully at 820x616 at 30fps (and up to 80fps), verified by saving frames and confirming non-zero pixel values.

### Lane Detection System — Full Architecture

After fixing the CSI camera, the lane detection system was rebuilt as a proper modular Python package. The structure separated concerns cleanly: config.yaml holds all tunable parameters including camera settings, HSV bounds, BEV source and destination points, PID gains, Stanley controller parameters, and stop line detection thresholds. config_loader.py implements YAML parsing into dataclasses with hot-reload support via file modification time checking and SIGHUP signal handling, allowing parameter tuning without restarting the process.

camera_capture.py wraps QCarCameras with a 20-frame warmup guard that waits for the sensor to stabilize before passing frames downstream, and attaches precise timestamps to each frame for latency profiling.

preprocessor.py implements the full image transformation chain: lens undistortion using the calibrated camera matrix, resize to 50% for processing speed, adaptive HSV thresholding where the V-channel minimum is automatically scaled based on frame mean brightness to handle lighting variation, ROI masking using a normalized trapezoid whose vertices are tunable from config, and bird's-eye perspective warp using the calibrated M matrix.

lane_detector.py implements sliding window search for cold start and warm-start search using the previous polynomial for frames where detection is already active, polynomial fitting with EMA smoothing at alpha=0.3 over recent fits, stop line detection using HoughLinesP on the warped binary image, a traffic signal callback hook for future integration, and lane width consistency validation rejecting fits where detected width deviates more than 30% from the expected value.

metrics.py computes curvature radius using the proper differential geometry formula and lateral offset in real-world meters by multiplying pixel offset by the calibrated xm_per_pix conversion factor.

controller.py implements both PID and Stanley steering controllers. The PID uses proportional, integral with anti-windup clamped at ±100, and derivative terms. Speed-adaptive gain scheduling reduces gains at higher speeds to prevent oscillation. An IMU dead reckoning fallback holds the last steering command for up to 2 seconds when lane detection is lost, using IMU angular velocity to estimate heading drift.

visualizer.py draws the green lane fill polygon, telemetry HUD, bird's-eye inset minimap, stop line indicator, and ego path centerline on the output frame.

All 22 unit tests across three test modules passed: 7 tests for config parsing, 9 tests for curvature and offset math, and 6 tests for preprocessor output shapes and lane pixel detection.

## SECTION 3: CHALLENGES FACED — WITH PROBLEM-SOLVING APPROACH

### Challenge 1: Lane Detection Failures Due to Yellow Tape Reflection

What the issue was: After the CSI camera was fixed, lane detection on the actual yellow tape track was unreliable. The detection pipeline would frequently report NONE or only one lane detected even when the camera had a clear view of both yellow lines. The confidence score would drop suddenly and the polynomial fit would jump erratically.

Root cause — HSV value mismatch: The yellow tape, when placed on a black plastic surface under lab fluorescent lighting, produced extremely atypical HSV readings. Measured values were H≈37, S≈30, V≈86 — a saturation of only 30 out of 255. Standard yellow in HSV has saturation above 100. The black surface was reflecting light back upward onto the tape, washing out its color saturation. A standard yellow HSV range of S:80–255 would miss this tape entirely.

Root cause — Perspective transform miscalibration: The source trapezoid for the bird's-eye warp was initially calibrated on a flat table at a fixed camera height and angle. When the car was placed on the floor with a different camera-to-ground geometry, the warped image no longer produced parallel lane lines. The sliding window search was therefore looking in the wrong pixel columns for the lane bases.

Root cause — Background clutter in the ROI: At the camera's field of view, the ROI trapezoid captured not just the lane ahead but also reflections off the black plastic sheet, shadows from the car body, and ambient floor texture. The binary image after thresholding contained false positives in these regions, causing the histogram peak detection to identify wrong lane base positions.

Approach taken for HSV: The HSV thresholds were widened significantly to accommodate low-saturation yellow: H:10–45, S:10–180, V:40–200. An adaptive V-channel compensation was added that automatically adjusts the V minimum based on the mean brightness of the current frame, so the threshold scales with lighting conditions rather than using a fixed value.

Approach taken for perspective calibration: A dedicated calibration script (calibrate_perspective.py) was built that captures a live frame, allows manual adjustment of the four source trapezoid vertices while displaying the resulting bird's-eye warp in real time, and saves the final verified values to config.yaml. This made recalibration repeatable without code changes.

Approach taken for background clutter: The ROI trapezoid was tightened to cover only the bottom 35% of the frame where the lane lines are consistently visible, reducing the area where false positives from reflections and shadows could affect the histogram. Morphological MORPH_CLOSE followed by MORPH_OPEN with a 3x3 kernel was applied after thresholding to close small gaps within lane lines while removing isolated noise pixels.

Why not use a deep learning lane detector like CondLaneNet or YOLOP: These models achieve very high accuracy on standard benchmarks but require either a pre-trained model fine-tuned on yellow tape on black surface, or training data collected from this specific track. Neither was feasible within the timeline. The classical CV pipeline with tunable parameters allowed rapid iteration on the specific track conditions.

Why RANSAC polynomial fitting over simple polyfit: Simple numpy polyfit fits a polynomial to all detected lane pixels with no outlier rejection. On real track frames with noise pixels and false positives from reflections, a single outlier cluster far from the true lane line can pull the polynomial significantly off. RANSAC randomly samples subsets, fits polynomials to each sample, counts inliers within a residual threshold, and returns the fit with the most support. This is substantially more robust to the kind of sparse noise seen from reflections on the black track surface.

### Challenge 2: LiDAR Not Stopping

What the issue was: When the main script was interrupted with Ctrl+C, the Python process exited but the physical LiDAR motor continued spinning. On the next program run, the LiDAR initialization would fail with error code -10 because the device was already in an active state from the previous session.

Root cause: Python's default KeyboardInterrupt handling does not guarantee that cleanup code in finally blocks runs completely, especially when background threads are blocked on hardware reads. The subprocess managing the lidar node was being sent SIGTERM but the Quanser PAL hardware layer was not being properly notified to release the physical device.

Approach taken: The proper hardware stop sequence from Quanser's own hardware_stop.py example was integrated into the shutdown path. This explicitly initializes a new QCarLidar() instance and calls .terminate() on it, which sends the correct hardware-level shutdown command to release the physical motor. This is separate from and in addition to killing the software process. The same pattern was applied to QCar(), ensuring the motor drive is also properly released on exit.

Why this approach: Quanser's own documentation and example hardware_stop.py uses this exact pattern. It is the vendor-recommended way to release hardware that has a physical state separate from the software process. Simply killing the process or sending SIGKILL does not flush the hardware command queue or release the DAQ driver.

### Challenge 3: FPS Below Target —

What the issue was: The initial perception pipeline ran at 2.1fps, well below the 15fps target needed for responsive obstacle avoidance.

Root cause: The bottleneck was YOLOv8 running on full 1280x720 frames. The model's inference time alone was over 400ms per frame at this resolution on the Orin.

Approach taken: The input resolution to the YOLO model was reduced from 1280 to 416 pixels (the imgsz parameter). The camera resolution for the perception node was also reduced to 640x360. Half-precision (FP16) inference was confirmed active since the hardware controller already set half=True when CUDA was available. This combination brought throughput from 1.9fps to approximately 4.2fps — a 2x improvement within the available time.

## Day 12 — Real-Time SDV Telemetry Dashboard

A full real-time telemetry dashboard was developed to provide a unified visual interface for monitoring all active sensors and vehicle state on the QCar2. The dashboard was built using HTML, CSS, and JavaScript on the frontend, with a FastAPI backend serving live sensor data. The interface runs in any browser over the local network, making it accessible from the development laptop without requiring any additional software installation.

The dashboard displays the following live feeds and data simultaneously:

All four CSI camera streams  front, rear, left, and right — rendered as individual video panels in a grid layout, each labeled with camera ID and showing the live image feed in real time. This gives a complete surround-view of the vehicle's environment from a single browser window.

The RealSense D435 depth camera feed  rendered as a colormap-overlaid depth image where closer objects appear in warmer colors and farther objects appear in cooler colors, making distance information immediately readable at a glance.

The RPLiDAR output  rendered as a live 2D polar plot showing all 360 scan points plotted in a coordinate frame centered on the vehicle. Each detected point is plotted at its measured angle and distance. The closest detected object's coordinates are highlighted separately, showing both its distance in meters and its angular position in degrees relative to the vehicle's forward axis.

Battery percentage displayed as a numeric readout and a visual bar indicator, pulled from the QCar2 hardware telemetry via the PAL library.

Steering angle displayed as both a numeric value in degrees and a visual arc indicator showing the current wheel angle, updated in real time from the vehicle's hardware state.